# Module -- Diagnostics Manager Core

# High-Level Description

# Figures

## Component Diagram

# Variable Data Dictionary

| Module Inputs | Module Outputs |  |
| --- | --- | --- |
| MEC_Cnt_enum |  |  |
| MfgDiagInhibit_Cnt_lgc |  |  |
| SystemState_Mode |  |  |

## Module Internal Variables

| Variable Name | Datatype | Resolution | Legal Range (min) | Legal Range (max) | Multiplicity | Software Segment {Data Type} |
| --- | --- | --- | --- | --- | --- | --- |
| NTCStrgArray_Cnt_str | NTCStrgArray | N/A | N/A | N/A | 1:1 | DIAGMGR_START_SEC_VAR_SAVED_ZONEHGS_UNSPECIFIED |
| NTCBlackBoxData_Cnt_str | NTCBlkBoxData | N/A | N/A | N/A | 1:1 | DIAGMGR_START_SEC_VAR_SAVED_ZONEHGS_UNSPECIFIED |
| DEMEventActive_Cnt_M_lgc[D_NUMOFDEMEVENTS_CNT_U08+1] | Boolean | N/A | FALSE | TRUE | 1:1 | DIAGMGR_START_SEC_VAR_CLEARED_BOOLEAN |
| ResetNTCFlag_Cnt_M_u08 | Refer * | Refer * | Refer * | Refer * | Refer * | Refer * |
| DiagMgr_NTCInfo#_Cnt_M_str | Refer * | Refer * | Refer * | Refer * | Refer * | Refer * |

### User defined typedef definition/declaration

| Typedef Name | Element Name | User Defined Type | Legal Range (min) | Legal Range (max) |
| --- | --- | --- | --- | --- |
| NTCStrgArray |  | NTCStrg |  |  |
| NTCBlkBoxData |  | NTCBlkBoxType |  |  |
| typedef struct { } NTCBlkBoxType | NTC_Cnt_u08 | Uint8 | 1 | FULL |
|  | Param_Cnt_u08 | Uint8 | 0 | FULL |
|  | SystemState_Cnt_u08 | Uint8 | 0 | 4 |
|  | VehSpd_Kph_u8p0 | Uint8 | 0 | FULL |
|  | BlkBoxCfgData1 | Uint32 | 0 | FULL |
|  | BlkBoxCfgData2 | Uint32 | 0 | FULL |
|  | BlkBoxCfgData3 | Uint32 | 0 | FULL |
|  | HwTrq_HwNm_s4p11 | Sint16 | -10 | 10 |
|  | MtrTrq_MtrNm_s4p11 | Sint16 | -8.8 | 8.8 |
|  | IgnCtr_Cnt_u16 | Uint16 | 0 | FULL |
| typedef struct { } NTCStrg | NTC | NTCNumber | 0 | 511 |
|  | Status | uint8 | 0 | FULL |
|  | AgingCounter | uint8 | 0 | FULL |
|  | Status | uint8 | 0 | FULL |
|  | AgingCounter | uint8 | 0 | FULL |

# Constant Data Dictionary

## Calibration Constants

Constant Name

k_FltRspTbl_Cnt_str[]

k_FltRmpRate_UlspmS_f32[]

## Program(fixed) Constants

### Embedded Constants

#### Local

| Constant Name | Resolution | Units | Value |
| --- | --- | --- | --- |
| D_FLTRSPNTCACTIVEBIT_CNT_B32 | N/A | Counts | 0x00800000 |
| D_FLTRSPRECOVERABLEBIT_CNT_B32 | N/A | Counts | 0x00400000 |
| D_FLTRSPHWASBSYSTMFLTBIT_CNT_B32 | N/A | Counts | 0x00200000 |
| D_FLTRSPDEFVEHSPDBIT_CNT_B32 | N/A | Counts | 0x00100000 |
| D_FLTRSPDEFTEMPBIT_CNT_B32 | N/A | Counts | 0x00080000 |
| D_FLTRSPSCOMHWANOTVALIDBIT_CNT_B32 | N/A | Counts | 0x00040000 |
| D_FLTRSPWIRDISABLEBIT_CNT_B32 | N/A | Counts | 0x00008000 |
| D_FLTRSPPWRCYCLTCHBIT_CNT_B32 | N/A | Counts | 0x00000010 |
| D_FLTRSPNTCINHIBITNOTOPERATEBIT_CNT_B32 | N/A | Counts | 0x00000020 |
| D_FLTRSPNTCINHIBITRUNBIT_CNT_B32 | N/A | Counts | 0x00000040 |
| D_FLTRSPRAMPBITS_CNT_B32 | N/A | Counts | 0x0000000F |
| D_FLTRSPBLKBOXBITS_CNT_B32 | N/A | Counts | 0x00003800 |
| D_BLKBOXBITOFFSET_CNT_U08 | N/A | Counts | 11 |
| D_RAMPNONE_CNT_U8 | 1 | Counts | 0x0F |
| D_RAMPF2_CNT_U8 | 1 | Counts | 0x0E |
| D_RAMPF1_CNT_U8 | 1 | Counts | 0x0D |
| D_DIAGRMPRTLOLMT_ULSPMS_F32 | Single precision float | Uls/mS | 0.0001 |
| D_DIAGRMPRTHILMT_ULSPMS_F32 | Single precision float | Uls/mS | 0.5 |
| D_FLTRESPIVTR1INACTV_CNT_U32 | N/A | Counts | 0x00010000UL |
| D_FLTRESPIVTR2INACTV_CNT_U32 | N/A | Counts | 0x00020000UL |

#### Global

Constant Name

D_NTCACTIVEBITS_CNT_B8

### Module specific Lookup Tables Constants

| Constant Name | Resolution | Value | Software Segment |
| --- | --- | --- | --- |
| T_NTCMapTbl#_Cnt_enum[SIZE] | N/A | Refer * | AP_DIAGMGR_CONST |
| T_DiagMgrNtcAppInfoMap_Cnt_Str[SIZE] | N/A | Refer * | AP_DIAGMGR_CONST |
| T_DiagMgrNtcInfoPtr_Cnt_Str[SIZE] | N/A | Refer * | AP_DIAGMGR_CONST |

Note: “ Refer *” -  Refer to Diagnostics_Manager_GeneratedCfg_MDD

Note Size and elements of Table constants varies across projects. Check project configuration files Under UTP/ Contract folder for data.

# Functions/Macros used by the Sub-Modules

## Library Functions / Macros

The library and functions / Macros that are called by the various sub modules are identified below,

- TableSize_m()

## Data Hiding Functions

- <None>

## Function Mapping

- DiagMgr_ReportDet(errorId) ↔  Det_ReportError(0,0,0,errorId)

## Global Functions/Macros Defined by this Module

### Diagnostic Manager Initialization

Note: *doc in below table is Diagnostics_Manager_GeneratedCfg_MDD

| Function Name | DiagMgr_Init_Core | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | NTCInfo_Cnt_T_str[SIZE] | Refer Range of DiagMgr_NTCInfo#_Cnt_M_str[SIZE] in *doc |  |  |
|  | NumElements_Cnt_T_u08 | Refer Range of DIAGMGR_EVENTNUM_# in *doc |  |  |
|  | CurrentAppIdx_Cnt_T_u08 | Refer Range of DIAGMGR_APID_# in *doc |  |  |
|  | DiagMgrInitComp_Ptr_T_lgc | Refer Range of DiagMgrInitComp#_Cnt_M_lgc |  |  |
|  | NTCInfoQueue_Cnt_T_str[5] | Refer Range of NTCInfoQueue#_Cnt_M_str[SIZE] in *doc |  |  |
|  | NTCQueueIndex_Ptr_T_u08 | Refer Range of NTCQueueIndex#_Cnt_M_u08 in *doc |  |  |
|  | DiagSts_Cnt_T_b16[SIZE] | Refer Range of DiagSts#_Cnt_M_b16[SIZE] in *doc |  |  |
|  | ActiveRmpRate_UlspmS_T_f32[SIZE] | Refer Range of ActiveRmpRate#_UlspmS_M_f32[SIZE] in *doc |  |  |
|  | ActDiagSts_Ptr_T_u08 | Refer Range of ActDiagSts#_Cnt_M_u08 in *doc |  |  |
| Return Value | none |  |  |  |

#### Description

### Diagnostic Manager Periodic Code

Note: *doc in below table is Diagnostics_Manager_GeneratedCfg_MDD

| Function Name | DiagMgr_Per_Core | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | NTCInfo_Cnt_T_str[SIZE] | Refer Range of DiagMgr_NTCInfo#_Cnt_M_str[SIZE] in *doc |  |  |
|  | NumElements_Cnt_T_u08 | Refer Range of DIAGMGR_EVENTNUM_# in *doc |  |  |
|  | DiagSts_Cnt_T_b16[SIZE] | Refer Range of DiagSts#_Cnt_M_b16[SIZE] in *doc |  |  |
|  | ActiveRmpRate_UlspmS_T_f32[SIZE] | Refer Range of ActiveRmpRate#_UlspmS_M_f32[SIZE] in *doc |  |  |
|  | ActDiagStsIdx_Ptr_T_u08 | Refer Range of ActDiagSts#_Cnt_M_u08 in *doc |  |  |
|  | T_NTCMapTbl_Cnt_enum[SIZE] | Refer range of T_NTCMapTbl#_Cnt_enum[SIZE] in *doc |  |  |
|  | PrevResetNTCFlag_Ptr_T_u08 | Refer Range of ResetNTCFlag#_Cnt_M_u08 in *doc |  |  |
| Return Value | none |  |  |  |

#### Description

### Diagnostic Manager Transition Core

Note: *doc in below table is Diagnostics_Manager_GeneratedCfg_MDD

| Function Name | DiagMgr_Trns_Core | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | NTCInfo_Cnt_T_str[SIZE] | Refer Range of DiagMgr_NTCInfo#_Cnt_M_str[SIZE] in *doc |  |  |
|  | NumElements_Cnt_T_u08 | Refer Range of DIAGMGR_EVENTNUM_# in *doc |  |  |
|  | T_NTCMapTbl_Cnt_enum[79] | Refer range of T_NTCMapTbl#_Cnt_enum[SIZE] in *doc |  |  |
| Return Value | none |  |  |  |

#### Description

### Diagnostic Manager Get NTC Failed

| Function Name | NxtrDiagMgr_GetNTCFailed_Core | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | NTC_Cnt_T_enum | NTCNumber | 0 | 511 |
|  | NTCFailed_Ptr_T_lgc | const boolean pointer | False | True |
| Return Value | RetVal | Std_ReturnType | E_NOT_OK | E_OK |

#### Description

### Diagnostic Manager Get NTC Active

| Function Name | NxtrDiagMgr_GetNTCActive_Core | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | NTC_Cnt_T_enum | NTCNumber | 0 | 511 |
|  | NTCActive_Ptr_T_lgc | const boolean pointer | FALSE | TRUE |
| Return Value | RetVal | Std_ReturnType | E_NOT_OK | E_OK |

#### Description

### Diagnostic Manager Get NTC Status

| Function Name | NxtrDiagMgr_GetNTCStatus_Core | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | NTC_Cnt_T_enum | NTCNumber | 0 | 511 |
|  | Status_Ptr_T_u08 | const boolean pointer | FALSE | TRUE |
| Return Value | RetVal | Std_ReturnType | E_OK E_NOT_OK |  |

#### Description

### Diagnostic Manager Set NTC Status

Note: *doc in below table is Diagnostics_Manager_GeneratedCfg_MDD

| Function Name | NxtrDiagMgr_SetNTCStatus_Core | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | NTC_Cnt_T_enum | NTCNumber | 0 | 511 |
|  | Param_Cnt_T_u08 | Uint8 | 0 | FULL |
|  | Status_Cnt_T_enum | NxtrDiagMgrStatus | NTC_STATUS_PASSED NTC_STATUS_FAILED NTC_STATUS_PREPASSED NTC_STATUS_PREFAILED |  |
|  | NTCInfo_Cnt_T_str[79] | Refer Range of DiagMgr_NTCInfo#_Cnt_M_str[SIZE] in *doc |  |  |
|  | DiagSts_Cnt_T_b16[2] | Refer Range of DiagSts#_Cnt_M_b16[SIZE] in *doc |  |  |
|  | ActiveRmpRate_Cnt_T_f32[2] | Refer Range of ActiveRmpRate#_UlspmS_M_f32[SIZE] in *doc |  |  |
|  | ActDiagSts_Ptr_T_u08 | Refer Range of ActDiagSts#_Cnt_M_u08 in *doc |  |  |
| Return Value | RetVal | Std_ReturnType | E_OK E_NOT_OK |  |

#### Description

### Diagnostic Manager Report NTC Status

Note: *doc in below table is Diagnostics_Manager_GeneratedCfg_MDD

| Function Name | NxtrDiagMgr_ReportNTCStatus_Core | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | NTC_Cnt_T_enum | NTCNumber | 0 | 511 |
|  | Param_Cnt_T_u08 | Uint8 | 0 | FULL |
|  | Status_Cnt_T_enum | NxtrDiagMgrStatus | NTC_STATUS_PASSED NTC_STATUS_FAILED NTC_STATUS_PREPASSED NTC_STATUS_PREFAILED |  |
|  | NTCInfoQueue_Cnt_T_str [5] | Refer Range of NTCInfoQueue#_Cnt_M_str[SIZE] in *doc |  |  |
|  | NTCQueueIndex_Ptr_T_u08 | Refer Range of NTCQueueIndex#_Cnt_M_u08 in *doc |  |  |
| Return Value |  | Std_ReturnType | E_OK |  |

#### Description

## Local Functions/Macros Used by this MDD only

### Set Bits

| Function Name | SetBits_u8 | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | Data | const uint8 pointer | 0 | 255 |
|  | BitMask | uint8 | 0 | 255 |
| Return Value | N/A |  |  |  |

#### Description

*Data |= BitMask

### Clear Bits

| Function Name | ClrBits_u8 | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | Data | const uint8 pointer | 0 | 255 |
|  | BitMask | uint8 | 0 | 255 |
| Return Value | N/A |  |  |  |

#### Description

*Data &= ~BitMask

### Read Bit

| Function Name | ReadBit_u8 | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | Data | uint8 | 0 | 255 |
|  | BitMask | uint8 | 0 | 255 |
| Return Value | (anonymous) | boolean | FULL | FULL |

#### Description

IF 0 = (Data & BitMask) THEN

return FALSE

ELSE

return TRUE

ENDIF

### Set Bits

| Function Name | SetBits_u16 | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | Data | const uint16 pointer | FULL | FULL |
|  | BitMask | uint16 | FULL | FULL |
| Return Value | N/A |  |  |  |

#### Description

*Data |= BitMask

### Read Bit

| Function Name | ReadBit_u32 | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | Data | uint32 | 0x000000 | 0xFFFFFF |
|  | BitMask | uint32 | 0x000000 | 0xFFFFFF |
| Return Value | (anonymous) | boolean | FULL | FULL |

#### Description

IF 0 = (Data & BitMask) THEN

return FALSE

ELSE

return TRUE

ENDIF

### Process Diagnostic Status

| Function Name | ProcessDiagSts | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | FltRsp_Cnt_T_u32 | Uint32 | 0 | FULL |
|  | DiagSts_Ptr_T_b16 | const uint16 pointer | 0 | FULL |
| Return Value | N/A |  |  |  |

#### Description

### Process Ramp Response

| Function Name | ProcessRampResponse | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | FltRsp_Cnt_T_u32 | Uint32 | 0 | FULL |
|  | ActiveRmpRate_Ptr_T_f32 | Const float32 pointer | 0.0001 | 0.5 |
|  | DiagSts_Ptr_T_b16 | Const uint16 pointer | 0 | FULL |
| Return Value | N/A |  |  |  |

#### Description

### Failed Check and Processing

| Function Name | FailedCheckAndProcessing | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | NTCInfo_Ptr_T_str | const uint16 NTCInfo_Str | See section 3.1.1 | See section 3.1.1 |
|  | DiagSts_Ptr_T_b16 | const uint16 pointer | 0 | FULL |
|  | MaxRampRate_Ptr_T_f32 | Const float32 pointer | 0.0001 | 0.5 |
|  | NTC_Cnt_T_enum | NTCNumber | 0 | 511 |
| Return Value | N/A |  |  |  |

#### Description

# Software Module Implementation

## Runtime Environment (RTE) Initial Values

| Data | Value |
| --- | --- |

## Initialization Functions

None

## Periodic Functions

None

## Fault Recovery Functions

None

## Shutdown Functions

None

## Interrupt Functions

None

## Fault Recovery Functions

None

## Shutdown Functions

None

## Interrupt Functions

None

## Serial Communication Functions

None

# Execution Requirements

## Execution Sequence of the Module

## Execution Rates for sub-modules called by the Scheduler

This table serves as reference for the Scheduler design

| Function Name | Calling Frequency | System State(s) in which the function is called |
| --- | --- | --- |

## Execution Requirements for Serial Communication Functions

| Function Name | Sub-Module called by (Serial Comm Function Name) |
| --- | --- |

# Memory Map Definition Requirements

## Sub Modules (Functions)

This table identifies the software segments for functions identified in this module.

| Name of Sub Module | Software Segment |
| --- | --- |

## Global and Local Functions

This table identifies the software segments for local functions identified in this module.

| Name of Sub Module | Software Segment |
| --- | --- |
| FailedCheckAndProcessing | AP_DIAGMGR_CODE |
| ProcessRampResponse | AP_DIAGMGR_CODE |
| ProcessDiagSts | AP_DIAGMGR_CODE |
| SetBits_u8 | AP_DIAGMGR_CODE |
| ClrBits_u8 | AP_DIAGMGR_CODE |
| ReadBit_u8 | AP_DIAGMGR_CODE |
| ReadBit_u32 | AP_DIAGMGR_CODE |
| SetBits_u16 | AP_DIAGMGR_CODE |

# Known Issues / Limitations With Design

- (Item #1)

# Revision Control Log

| Rev # | Change Description | Date | Author Initials |
| --- | --- | --- | --- |
| 1 | Initial MDD version | 14-Feb-13 | VK |
| 2 | MDD catch up to match latest SRC Ver 4 | 24- June- 13 | NRAR |
| 3 | Updates/cleanup while adding latch active diagnostic | 5-OCT-13 | Jared |
| 3 | Updates to MDD done as per UTP fixes Ver is still taken as 3 to match with synergy ver | 19-Mar-14 | KPIT-PM |
| 4 | Updated for Inverter1 and Inverter 2 Fault response | 20-July-15 | Selva |